# Chapter 2: State of the Art in Ultra-Low-Power Capacitor-Less Voltage Regulators

# 2.1. Introduction to the Power Management Paradigm

The unprecedented proliferation of always-on Internet of Things (IoT) nodes, biomedical implantable devices, and microcontrollers due to the recent rise of software-defined-vehicles has catalyzed a paradigm shift in the design of fully integrated power management integrated circuits (PMICs). At the core of these embedded systems lies the Low-Dropout (LDO) or linear voltage regulator(VREG), a pivotal component responsible for providing a clean, stable, and precise supply voltage to sensitive downstream analog and digital blocks. Historically, LDOs and VREGs relied heavily on a bulky external output capacitor, typically in the range of 1µF to 10µF, to ensure loop stability and to buffer charge during sudden load transients.

However, the modern drive toward extreme miniaturization and cost reduction in System-on-Chip (SoC) products favorizes the complete elimination of off-chip components. Removing the external capacitor translates directly to a reduced Bill of Materials (BOM) and a minimized printed circuit board (PCB) footprint. This architectural evolution has given rise to the Output-Capacitor-Less (OCL) or capless voltage regulator.

While advantageous for integration, removing the output capacitor introduces a fundamental tri-lemma in analog design: balancing ultra-low quiescent current IQ, fast transient response, and high static precision across all process, voltage, and temperature (PVT) variations. When the quiescent current must be aggressively scaled down to the sub-microampere or low-microampere domain to extend battery life, the internal error amplifier’s bandwidth and slew rate are severely degraded. Consequently, achieving acceptable transient undershoots and overshoots during full-load current steps (eg: 100nA to 10mA) becomes an exceptionally complex challenge. The following sections explore the established research directions and state-of-the-art topologies developed to address these inherent tradeoffs.

# 2.2. Frequency Compensation and Loop Stability in OCL Architectures

In classical VREG architectures, the large external capacitor establishes a dominant, low-frequency pole at the output node. Because this pole resides at a much lower frequency than the internal poles of the error amplifier (EA), the system inherently behaves as a single-pole system up to its unity-gain frequency (UGF), ensuring a robust phase margin, especially under medium and heavy load conditions. For light loads, the internal poles might approach the UGF, but usually, the ESR of the output capacitor provides a beneficial zero that helps maintain stability.

When the output capacitor is removed (leaving only the parasitic capacitance of the load, typically in the range of 0 to a couple nFs at most), the output pole is pushed to a much higher frequency. More critically, the frequency of this output pole becomes highly load-dependent. At minimal load currents, the transconductance gm of the pass device is exceedingly small, causing pout to drift closer to the internal poles of the EA. This proximity results in complex conjugate poles, leading to severe peaking in the frequency response and catastrophic instability.

To resolve this, early state-of-the-art implementations focused on advanced frequency compensation schemes. A foundational methodology was introduced by Leung and Mok [3], who proposed the Damping-Factor-Control (DFC) frequency compensation. DFC introduced an active zero to dynamically track the shifting output pole, ensuring wideband stability without relying on load capacitance. While pioneering, classic DFC and Miller compensation techniques often require large internal compensation capacitors, consuming valuable silicon area.

Modern approaches favor the generation of adaptive or load-tracking zeros. By inserting a variable resistor—often implemented via a MOSFET operating in the linear or saturation region whose gate is controlled by a replica of the load current—a zero can be synthesized that naturally shifts in tandem with the output pole. This method effectively flattens the phase response at the unity-gain crossover frequency. However, tracking zeros must be meticulously designed to account for worst-case PVT corners, as an over-compensated zero can lead to early phase dropping, while an under-compensated zero fails to mitigate complex pole formation.

# 2.3. Power Stage Architectures: The Shift from PMOS to NMOS

The choice of the power pass-device dictates the fundamental topology of the regulator, directly impacting both the dropout voltage and the transient capabilities of the system.

## 2.3.1. PMOS-Based Power Stages

The vast majority of classical LDOs and many recent ultra-low-power architectures utilize a PMOS transistor as the power pass-device, configured as a common-source amplifier. The primary advantage of a PMOS stage is its ability to operate with an extremely low dropout voltage, meaning the input supply voltage can be nearly identical to the required output voltage.

Recent literature showcases impressive ultra-low power PMOS implementations. Yao et al. [5] achieved an external capacitor-less LDO with a maximum quiescent current of 2.3&mu;A for a 1mA load. Similarly, Zhan and Ki [7] demonstrated a PMOS LDO capable of supporting a 12mA load while maintaining a 28.6&mu;A standby current. Furthermore, Tan et al. [8] developed a highly optimized 30mA PMOS LDO with a hybrid adaptive biasing scheme, maintaining just 7&mu;A of I_Q.

Despite these achievements, PMOS power stages suffer from three drawbacks in VREG capless designs. First, the PMOS seen from the supply side builds a common gate stage, which severly impacts the PSRR performance at middle and higher frequencies. As a common-source stage, as seen from the OTA side, the PMOS power stage exhibits a high intrinsic output impedance (r_ds), which pushes the load-dependent pole to lower frequencies, exacerbating stability issues. Second, because holes have lower mobility than electrons, a PMOS device must be sized considerably larger than an equivalent NMOS device to drive the same maximum load current. This increased silicon area also results in a bigger gate capacitance (C_gs + C_gd).

## 2.3.2. NMOS-Based Power Stages

To overcome the intrinsic bandwidth and slew-rate limitations of PMOS devices, advanced research directions have increasingly adopted NMOS power stages [6], [10]. Configured as a source-follower, the NMOS pass-device provides an intrinsically low output impedance (1/g_m).

Furthermore, the superior electron mobility of the NMOS transistor allows it to be physically smaller than a PMOS device for the same current rating (e.g., 10mA to 50mA). Consequently, the parasitic gate capacitance is drastically reduced, allowing the low-power internal amplifier to slew the gate node much faster. For instance, Liu et al. [6] demonstrated an area-efficient capless NMOS LDO with exceptionally fast transient response metrics.

The primary tradeoff of the NMOS architecture is the threshold voltage ($V_{th}$) penalty. Because it operates as a source follower, the gate voltage must be driven significantly higher than the output voltage. This typically requires either a higher secondary input supply rail or an internal charge pump if the VREG needs to be used as an LDO. In applications where multiple voltage rails are already present (such as stepping down from a 2.5V rail to a 1.5V core voltage), the NMOS topology presents a superior solution for transient-optimized capless regulation.

# 2.4. Overcoming Slew-Rate Bottlenecks at Nano-Ampere Bias

When the maximum allowed standby current is restricted to the 100 nA to 10 μA range, conventional linear error amplifiers cannot generate enough current to rapidly alter the gate voltage of the pass device during a transient event. The state-of-the-art literature has explored three predominant methodologies to break the correlation between static bias current and dynamic slew rate.

## 2.4.1. Adaptive Biasing

Rather than operating the internal amplifiers at a fixed, statically low bias current, adaptive biasing circuits dynamically adjust the internal IQ in proportion to the output load current. At minimal loads (e.g., 100 nA), the regulator consumes only a baseline, microampere-level current, maximizing light-load power efficiency. As the load current increases toward its maximum, a current-sensing mechanism scales up the internal bias. This dynamic scaling significantly increases the transconductance of the internal amplifiers, boosting the unity-gain frequency and extending the stability phase margin at heavy loads [7], [8]. While highly effective in steady-state heavy-load conditions, pure adaptive biasing struggles during the initial moments of a sudden positive load step (e.g., 100 nA → 10 mA). At the exact moment the load jumps, the internal bias is still in its ultra-low state, resulting in a delayed reaction and significant initial undershoots.

## 2.4.2. Slew-Rate Enhancement (SRE) Circuits

To address the instantaneous reaction gap of adaptive biasing, researchers have heavily utilized parallel Slew-Rate Enhancement (SRE) feedback loops. SRE mechanisms act as transient bypass paths. A classic implementation, demonstrated by Zhan and Ki [2], involves subthreshold undershoot-reduction techniques utilizing edge detectors.

In these architectures, a high-pass filter—usually realized via a series coupling capacitor—is connected between the regulator’s output and auxiliary driving circuitry. Under steady-state DC conditions, the capacitor blocks the signal, and the SRE is inactive, consuming zero power. However, during a fast load transient, the sudden drop in Vout (high dV/dt) passes through the coupling capacitor, instantly turning on the auxiliary circuitry to inject a massive surge of current into the pass device’s gate. Recent ultra-low power papers [9], achieving quiescent currents as low as 10 nA, rely extensively on complex SREs to reduce transient undershoots. The crucial disadvantage of SRE topologies is the silicon area. The required on-chip coupling capacitors are physically large.

## 2.4.3. Dynamic Current Scaling via Nonlinear Loops and Current Conveyors

To achieve massive slew rates without consuming the silicon area of SRE coupling capacitors, contemporary research has pivoted toward dynamically scaling Class-AB architectures and translinear circuits. Galan et al. [4] introduced the concept of super Class-AB Operational Transconductance Amplifiers (OTAs). These amplifiers utilize nonlinear characteristics—exploiting the transition between the cutoff, triode, and saturation regions of MOSFETs—to provide transient currents that are exponentially larger than their baseline bias current.

Expanding on this philosophy, the use of Second-Generation Current Conveyors (CCII) has emerged as a robust solution. A CCII is fundamentally a current-feedback amplifier. Unlike classical voltage-feedback OTAs whose slew rates are strictly limited by their tail currents, a CCII allows for a nearly unbounded transient current drive. By combining a CCII with nonlinear current mirrors—where specific mirroring transistors are dynamically forced from the triode region into saturation during transient events—the regulator can achieve a massive surge in dynamic drive current without any capacitive coupling. This approach directly solves the transient bottleneck while maintaining a compact silicon footprint.

# 2.5. Methodologies for High Static Precision and DC Gain

Beyond transient response, an automotive-grade VREG must maintain excellent static precision. In high-performance SoCs, the load regulation (the variation in Vout across the full load current spectrum) must typically be kept below a few millivolts. To achieve less than 2mV of variation for a 10mA load jump, the regulator’s closed-loop control system requires an exceptionally high open-loop DC gain, typically exceeding 70 dB.

Two primary research paths have addressed this:

- Cascoded Architectures: The traditional approach to boosting gain is stacking transistors in a cascoded or double-cascoded configuration. This multiplies the output impedance of the amplifier stages. The limitation is the reduction in voltage headroom, making this technique challenging for sub-1V applications. However, for regulators operating from a 2.5V supply, double-cascoded OTAs remain the gold standard for robust, mismatch-resilient, high-gain amplification.

- Positive Feedback and Negative Resistance: To boost gain without sacrificing voltage headroom, Oh and Bakkaloglu [1] introduced a radical approach utilizing cross-coupled positive feedback. By engineering a negative resistance loop, the output conductance of the amplifier is theoretically cancelled out, resulting in near-infinite low-frequency DC gain. While this current-reused gain-enhancement path significantly improves static precision, it inherently introduces the severe risk of localized loop instability. Small mismatches over PVT corners or over MC300 runs can easily cause the positive feedback to overpower the negative feedback, leading to oscillation.

# 2.6. Positioning and Justification of the Proposed Architecture

An analysis of the state-of-the-art reveals a fragmented landscape: PMOS designs achieve ultra-low IQ but suffer from degraded PSRR and increased silicon area; SREs provide fast recovery but consume extra silicon area; and negative-resistance techniques yield high precision but risk instability. The master’s thesis presented herein addresses these conflicting tradeoffs through a highly optimized, nonlinear dual-loop 1.5V capless voltage regulator.

Designed to convert a 2.5V supply to a highly stable 1.5V output, the proposed architecture selectively aggregates the most robust state-of-the-art techniques while discarding their respective compromises:

- Dual-Loop Separation of Concerns: To be able to optimize static precision and dynamic response separately without trying to get both out of a single OTA a dual loop architecture was chosen. The outer regulation loop employs a double-cascoded symmetrical OTA. Leveraging the adequate 2.5V headroom, this stage safely achieves over 80 dB of DC gain at loads above 200 μA, ensuring 2 mV maximum load regulation in Monte Carlo analyses.

- Nonlinear CCII Inner Loop: Instead of relying on area-consuming capacitively coupled SREs [2], transient robustness is governed by a parallel inner loop based on a CCII. The novelty lies in its coupling with a nonlinear PMOS current mirror (exploiting an emulated resistor, R_dyn). Mirroring the behaviour from [4], R_dyn glides from the triode region into deep saturation during a sudden output voltage undershoot. This drastically multiplies the positive slew rate dynamically, ensuring maximum undershoots of only 221 mV during massive 100 nA → 10 mA full-load steps.

- NMOS Power Stage and Stability: By selecting an NMOS power stage [6], the design capitalizes on intrinsically low output impedance to push the dominant output pole to higher frequencies and increased PSRR. Loop stability—capable of accommodating 0 to 10 nF of capacitive loading is guaranteed through a variable resistor that generates a continuous load-tracking zero, adapting to the output pole’s movement across the entire current spectrum.

- Adaptive + PTAT Biasing: Finally, to preserve battery life without starving the amplifiers, the system employs a PVT-invariant PTAT (Proportional to Absolute Temperature) base biasing current coupled with adaptive biasing [7], [8]. This guarantees a highly competitive worst-case (Fast, 85°C) standby current of just 2.1 μA at minimum load (100 nA), scaling up efficiently as the load approaches 10 mA and as the ambient temperature increases.

In conclusion, by strategically fusing NMOS wideband stability, dual-loop gain compartmentalization, and nonlinear current-conveyor dynamic scaling, the proposed regulator directly addresses the fundamental tradeoffs of ultra-low-power PMIC design, positioning itself as a robust, area-efficient, and highly precise solution relative to current literature.

## 2.7 Bibliography / References

[1] W. Oh and B. Bakkaloglu, "A CMOS Low-Dropout Regulator With Current-Reused Gain-Enhanced Feedforward Path," IEEE Transactions on Circuits and Systems II: Express Briefs, vol. 60, no. 11, pp. 756-760, Nov. 2013.

[2] C. Zhan and W.-H. Ki, "An Output-Capacitor-Free Adaptively Biased Low-Dropout Regulator With Subthreshold Undershoot-Reduction for SoC," IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 59, no. 5, pp. 1119-1131, 2012.

[3] K. N. Leung and P. K. T. Mok, "A capacitor-free CMOS low-dropout regulator with damping-factor-control frequency compensation," IEEE Journal of Solid-State Circuits, vol. 38, no. 10, pp. 1691-1702, Oct. 2003.

[4] J. A. Galan, A. J. Lopez-Martin, R. G. Carvajal, J. Ramirez-Angulo, and C. Rubia-Marcos, "Super Class-AB OTAs With Adaptive Biasing and Dynamic Output Current Scaling," IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 54, no. 3, pp. 449-457, March 2007.

[5] Y. Yao, M. Zhao, and X. Wu, "A Max 2.3μA Quiescent Current External Capacitorless Low-Dropout Regulator," in IECON 2020 - 46th Annual Conference of the IEEE Industrial Electronics Society, Singapore, 2020.

[6] X. Liu, C. Zhan, and H. Qiao, "Chip-Area-Efficient Capacitor-less LDO Regulator with Fast-Transient Response," in 2019 IEEE International Conference on Integrated Circuits, Technologies and Applications (ICTA), Chengdu, China, 2019.

[7] C. Zhan and W.-H. Ki, "Analysis and Design of Output-Capacitor-Free Low-Dropout Regulators With Low Quiescent Current and High Power Supply Rejection," IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 61, no. 2, pp. 625-636, Feb. 2014.

[8] Y.-C. Tan et al., "A 30-mA Compensation-Free Output-Capacitorless Analog LDO With Hybrid Adaptive Biasing," IEEE Transactions on Power Electronics, vol. 40, no. 8, pp. 11182-11196, Aug. 2025.

[9] M. Huang, Y. Lu, and R. P. Martins, "A 10 nA ultra-low quiescent current and 60 ns fast transient response low-dropout regulator for Internet-of-Things," IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 69, no. 1, pp. 433-442, 2022.

[10] N. Yan, J. Jiang, and H. Min, "A 12 nA Ultra-Low Quiescent Current Capacitor-Less LDO with 350 ns Fast Transient Response," IEEE Transactions on Circuits and Systems II: Express Briefs, vol. 69, no. 3, pp. 799-803, 2022.