# Chapter 3: Theoretical Fundamentals

In this chapter, the main specifications of linear regualtors will be discussed

Classical Voltage Regulator Topology: a) with PMOS pass device and b) with NMOS pass device

## Static/Steady-State Parameters

### Drop-out Voltage

The drop-out voltage is defined as the minimum voltage difference between VIN and VOUT (Figure 2.1) to maintain an intended voltage regulation [4_yang_li_thesis]. A linear regulator which can operate in low drop-out voltage regulation is commonly named as low drop-out regulator (LDO). The application intended for the proposed voltage regulator has quite a big dropout, that allows us to use an NMOS pass device without a charge pump, as such it does not qualify it as a Low-Dropout Regulator.

### Quiescent Current

The quiescent current (IQ) is defined as the total current drawn by the voltage regulator from the preceding supply, minus the current that flows to the load. In other words it’s the current consumed by the control circuits. The power efficiency &eta; of a linear regulator can be expressed in, where ILoad is current flowing into the load and Vdropout is the dropout voltage.

η=PdeliveredPsupplied=VoutIloadVin(Iload+IQ)=1−VdropoutVinIload(Iload+IQ)

### Line Regulation

Line regulation measures a voltage regulator’s ability to maintain a stable output voltage Vout despite static or low-frequency variations in its input supply voltage Vin. It is quantitatively defined as the ratio of the change in output voltage to the change in input voltage:

Line Regulation=ΔVoutΔVin=voutvin

Note: This is also frequently expressed as a percentage of the nominal output voltage per volt, i.e., %/V, or simply in mV/V.

The input supply feeding the regulator is rarely perfectly stable. It often fluctuates due to a combination of factors, such as:

- The line and load regulation limitations of the preceding supply regulator.

- Thermal drift and bandgap reference inaccuracies.

- Static offsets resulting from transistor mismatches.

- AC ripple originating from upstream switching regulators or rectifiers.

Ideally, the regulated output voltage should remain constant at its target value across the entire operating range of the input voltage. This ideal behavior holds true until the input voltage drops too close to the output voltage, reaching the "dropout" threshold. For an NMOS pass-transistor topology, as Vin decreases toward this limit, the gate-driving stage (often an error amplifier powered by Vin or a charge pump) runs out of voltage headroom to sufficiently command the NMOS gate. This pushes the NMOS pass element from its active (saturation) region into the triode region, causing the regulator to drop out of regulation.

However, even within the normal operating range, the output voltage is not fully immune to input fluctuations. Due to the finite loop gain of the feedback network, the output voltage varies slightly in response to changes in the input supply.

Specifically, the closed-loop line regulation is determined by the open-loop sensitivity of the regulator divided by the feedback loop gain:

∂Vout∂Vinclosed_loop≈∂Vout∂Vinopen_loop1+T(s)

where T(s) represents the loop gain of the regulation loop (the product of the error amplifier gain, the pass element gain, and the feedback factor).

In an open-loop condition, a change in Vin directly affects the output through the finite output impedance (rds) and channel-length modulation of the pass transistor, as well as through any body effect (particularly in NMOS source followers). Because the loop gain T(s) is finite, and further decreases at higher frequencies, the error amplifier cannot completely suppress these perturbations, leading to a measurable shift in Vout when Vin changes. To achieve superior line regulation, the low-frequency gain of the error amplifier must be maximised.

#### Small-Signal Analysis of the Line Regulation of a voltage regulator with an NMOS Pass device

To establish a formal quantitative analysis of the line regulation for an NMOS-based regulator, we evaluate its small-signal model. The analysis starts from the regulator model of classical_voltage_reg_topologies_nmos_and_pmos_pass_devices(b). Let the transconductance of the NMOS pass transistor MN be gmn, its output resistance be rds_n, and the open-loop voltage gain of the error amplifier be AOTA.

The feedback network consists of a resistive divider R1 and R2, while RL represents the load resistance. The small-signal relationship describing the propagation of an input variation ΔVin to the output variation ΔVout is given by:

−ΔVoutR2R1+R2AOTA−ΔVoutgmn+ΔVin−ΔVoutro,n(R1+R2)∥RL=ΔVout

By solving this equation for the transfer function ΔVout/ΔVin, the exact expression for the line regulation of the NMOS regulator is derived as:

ΔVoutΔVin=voutvin=11+R2R1+R2AOTAgmnro,n+ro,n(R1+R2)∥RL+1

Under typical operating conditions, the loop gain is high, and the expression could be simplified to:

ΔVoutΔVin=voutvin≈1R2R1+R2AOTAgmnro,n

### Physical Comparison: NMOS vs. PMOS Regulators

The simplified formulation reveals that the line regulation of the NMOS regulator is inversely proportional to both the amplifier’s open-loop gain AOTA and the intrinsic gain of the NMOS pass transistor, gmnro,n.

Compared to a conventional PMOS-based regulator—which typically exhibits a line regulation of approximately:

ΔVoutΔVinPMOS≈1R2R1+R2AOTA

the NMOS regulator improves line regulation performance by a factor of gmnro,n.

This key performance advantage is physically intuitive when considering the topology of the pass device. The NMOS pass transistor effectively functions as a "cascode" transistor situated between the noisy input supply Vin (connected to its drain) and the output node Vout (connected to its source). Because any voltage variation applied to the drain of a regulated NMOS transistor is inherently shielded by the channel, its equivalent variation at the source node is heavily attenuated by the transistor’s intrinsic gain gmnro,n in addition to the closed-loop feedback gain AOTA.

## Load Regulation

Load regulation measures a voltage regulator’s ability to maintain a stable output voltage in the presence of static or low-frequency variations in the load current. It is mathematically defined as the ratio of the static output voltage variation (ΔVout) to the static load current change (ΔIout or ΔIload):

Load Regulation=ΔVoutΔIload=voutiload

This ratio essentially represents the closed-loop output impedance (Rout_closed_loop) of the regulator. A lower closed-loop output impedance corresponds to superior load regulation, meaning lower static output voltage deviations due to varying load currents.

### Small-Signal Analysis of the NMOS Regulator

To analyze the load regulation of the NMOS regulator quantitatively, we utilize the small-signal representation based again on the regulator model from classical_voltage_reg_topologies_nmos_and_pmos_pass_devices(b). In this model, AOTA is the open-loop gain of the error amplifier, gmn is the transconductance of the NMOS pass element, and Iload represents the load current flowing through the load resistance RL.

When the load current changes by a small-signal amount ΔIload, it induces a corresponding shift in the output voltage ΔVout. The small-signal relationship describing the impact of this load current variation on the output voltage is formulated as:

−ΔVoutR2R1+R2AOTA−ΔVoutgmn=ΔIload

By rearranging the terms to isolate the transfer function ΔVout/ΔIload, we obtain the closed-loop expression:

ΔVoutΔIload=−1R2R1+R2AOTA+1gmn

Under typical operating conditions where the loop gain is significantly high (such that R2R1+R2AOTA≫1), this expression simplifies to:

ΔVoutΔIload≈−1R2R1+R2AOTAgmn

The negative sign indicates that an increase in load current (current drawn out of the regulator) causes a small, proportional drop in the output voltage.

### Physical Intuition and Feedback Mechanism

The small-signal analysis demonstrates that the load regulation of the NMOS regulator depends on both the amplifier open-loop gain (AOTA) and the transconductance of the NMOS pass device (gmn).

This dependency is directly linked to the properties of negative shunt feedback:

- Open-loop Output Impedance: The pass device in an NMOS regulator behaves as a source follower. Looking into the source of the NMOS transistor, the open-loop output impedance is approximately:

Rout,ol≈1gmn

- Loop Gain: The feedback loop consists of the resistor divider ratio β=R2/(R1+R2), the error amplifier gain AOTA, and the pass-device source-follower gain (which is approximately unity). This yields a loop gain of:

T≈R2R1+R2AOTA

- Closed-loop Output Impedance: The voltage-sensing shunt-feedback at the output node reduces the output impedance by a factor of 1+T:

Rout,cl=Rout,ol1+T≈1/gmnR2R1+R2AOTA=1R2R1+R2AOTAgmn

This derivation confirms that to minimize output voltage variations under fluctuating load conditions, the design should focus on maximizing both the error amplifier’s open-loop gain (AOTA) and the transconductance (gmn) of the NMOS pass element.

## Temperature Drift

Temperature variations directly affect the output voltage accuracy of a linear regulator. The thermal stability of a regulator is quantified by its temperature coefficient (TC), which is defined as the fractional change of the output voltage in response to a change in temperature [5 G. Rincon-Mora, Analog IC Design with Low-Dropout Regulators]. Typically expressed in percent per degree Celsius (%/∘C) or parts per million per degree Celsius (ppm/∘C), the temperature coefficient is mathematically defined as:

TC=1VoutΔVoutΔT=(ΔVref+ΔVos)VoutVrefVoutΔT=ΔVref+ΔVosVrefΔT

Where:

- ΔT represents the change in temperature.

- ΔVref represents the variation in the reference voltage over temperature.

- ΔVos represents the variation in the error amplifier’s input-referred equivalent offset voltage over temperature.

Based on the derivation above, the absolute value of the output voltage cancels out. This indicates that the regulator’s overall temperature coefficient is independent of the target output voltage setting. Instead, the thermal stability of the regulator is directly determined by two primary components:

- The reference voltage temperature coefficient: ΔVrefVrefΔT

- The input-referred offset voltage drift scaled by the reference voltage: ΔVosVrefΔT

To minimize temperature drift in practical linear regulator designs, it is necessary to utilize a highly stable bandgap reference with a low temperature coefficient, alongside an error amplifier optimized to minimize thermal offset drift (ΔVos/ΔT) through a low mismatch optimized input pair, high DC open-loop gain and layout matching techniques.

## Deep Dive: Minimizing Thermal Drift in Linear Regulators

Achieving high-precision thermal performance in linear regulators requires addressing both the primary reference voltage source and the non-idealities of the feedback loop. This section examines the specific design and layout techniques used to mitigate these thermal effects.

### 1. Bandgap Reference (BGR) Stability and Curvature Correction

The bandgap reference provides the baseline voltage Vref. Standard bandgap circuits achieve first-order temperature cancellation by summing two voltages with opposite thermal behaviors:

- CTAT (Complementary-to-Absolute-Temperature): The base-emitter voltage of a bipolar junction transistor (VBE) decreases linearly with temperature, typically at a rate of approximately:

∂VBE∂T≈−2 mV/∘C

- PTAT (Proportional-to-Absolute-Temperature): The thermal voltage VT=kBT/q increases linearly with temperature, exhibiting a temperature coefficient of:

∂VT∂T≈+0.086 mV/∘C

By scaling and summing these two voltages, a first-order temperature-compensated reference voltage of approximately 1.2 V (or lower for fractional bandgaps) is produced:

Vref(T)=VBE(T)+K⋅VT(T)

where K is a constant defined by transistor sizing and resistor ratios.

#### Curvature Correction

In high-precision applications, first-order cancellation is insufficient because VBE possesses a non-linear term proportional to Tln(T). This non-linearity results in a parabolic (curved) TC profile over a wide temperature range, typically limiting standard references to 20 to 50 ppm/∘C.

To achieve sub-5 ppm/∘C drift, designers employ curvature correction techniques. These circuits generate a non-linear current component—often proportional to T2 or PTAT2—and inject it back into the summation node. This effectively flattens the parabolic curve, canceling the high-order temperature dependencies of VBE.

### 2. Mitigating Error Amplifier Offset Drift

Even with a perfectly stable reference, any thermal drift in the error amplifier’s input-referred offset voltage (ΔVos/ΔT) propagates directly to the output. This offset is composed of both random and systematic components.

#### Random Offset and Pelgrom’s Law

Random offset arises from local, microscopic variations in transistor physical dimensions, doping concentrations, and oxide thicknesses. In the input differential pair of the error amplifier, threshold voltage (Vth) mismatch is the dominant driver of offset drift.

According to Pelgrom’s Law, the standard deviation of threshold voltage mismatch (σΔVth) is inversely proportional to the square root of the active gate area:

σΔVth=AVthW⋅L

where AVth is a technology-dependent constant, while W and L represent the width and length of the input transistors.

To minimize random thermal drift: * Large Device Sizing: Designers use large gate areas (high W⋅L) for the input pair. Increasing the area averages out local channel fluctuations, lowering both the nominal offset and its corresponding drift over temperature. * PMOS vs. NMOS: PMOS input pairs are often preferred in sub-micron processes due to lower flicker (1/f) noise and historically more predictable matching parameters compared to NMOS equivalents.

#### Systematic Offset and High DC Open-Loop Gain

Systematic offset occurs due to circuit design asymmetries, such as drain-to-source voltage (VDS) imbalances in the input pair or load current mirrors. Because these biases change dynamically with temperature, they introduce systematic drift.

Maintaining a high DC open-loop gain (AOL) in the error amplifier minimizes this systematic drift by heavily attenuating closed-loop gain errors:

ϵgain≈11+AOLβ

A high AOL ensures that the amplifier behaves closer to its mathematical ideal, ensuring that thermal variations in internal node voltages do not manifest as a shifting input-referred offset.

### 3. Layout Matching and Thermal Gradient Mitigation

Even the most robust circuit topology can suffer from severe temperature drift if the physical layout is not optimized. Silicon chips experience localized thermal gradients, especially in linear regulators where the power pass transistor dissipates significant heat.

#### Isothermal Contours and Spatial Placement

When the pass transistor handles high currents, it acts as a localized heat source, projecting heat outward in concentric waves. If one transistor of the error amplifier’s differential input pair is even slightly closer to this heat source than the other, a temperature difference (ΔTinput) is established between them. Because transistor parameters are highly temperature-dependent, this spatial temperature difference translates directly into a severe, thermally-induced offset drift.

To mitigate this: * Symmetrical Placement: The error amplifier’s input pair is placed along isothermal contours (lines of constant temperature). Designers place the input pair as far as possible from the pass element, oriented perpendicularly to the heat dissipation path.

#### Common-Centroid and Cross-Coupled Geometries

To cancel out linear spatial gradients (such as variations in packaging stress, doping concentrations, and temperature), the input pair is split into multiple parallel sub-transistors and cross-coupled in a common-centroid layout (e.g., an ABBA or diagonal quadrant configuration).

### Precision Calculation

Parameter Vout_static Typical 1.5 &#177; 3% → min: 1.455V, max: 1.545V

Vref_ideal = 1.2V

## Transient Response and Slew Rate

The Slew Rate directly impacts the transient response of the voltage regulator, which is crucial for maintaining the output voltage within acceptable limits under varying load conditions. In automotive applications, where power efficiency and reliability are paramount, optimizing the Slew Rate can lead to improved performance and reduced power consumption.

Usually, for a step in the output load current, from 0 to a maximum value Iload_max, the maximum output voltage variation ΔVout_max can be expressed as:

ΔVout_max=(1BWcl+ΔVgate⋅CgateISR)⋅Iload_maxCload

At the first moment of the current step, the control loop has not had time to react. The extra current demand, ΔIload_max, is supplied entirely by the output capacitor, Cload. This causes the output voltage to start dropping at a rate: ΔIload_maxCload. The output voltage will stop dropping at the instant when the power stage supplies exactly the same value of the demanded load current. Until then, the loop will try to react and restore the output voltage, but the response time is limited by the loop bandwidth, BWcl. During this time, the output voltage will have dropped by an amount: ΔIload_maxCload⋅BWcl. After the loop starts to react, the gate voltage of the pass transistor will begin to change. The time it takes for the gate voltage to change by the required amount such as to supply the demanded load current can be written as: tsr=ΔVgateSRGate=ΔVgate⋅CgateISR.

As can be seen from the equation the loop bandwidth and the slew rate of the OTA play an essential role in shaping the transient response of the voltage regulator.

If we consider the NMOS voltage regulator as a two-pole system with the dominant pole at the power stage’s node and the second pole at the output node, the worst case stability scenario is at low load current and high output capacitance. The value of the gate capacitor should be chosen such that stability at the lowest required load current and maximum load capacitance is ensured. From this design constraint, there is an effective upper bound on the possible bandwidth of the control loop.

The other parameter that we are free to tune is the Slew Rate of the OTA. Indeed, increasing the Slew Rate will reduce the time it takes for the gate voltage to change, thus improving the transient response of the regulator.

loadJump_ota_model_linear_transientResponse_2u_vs_5u_Islew shows the difference in transient response between two OTA models with different Slew Rates. The OTA with a higher Slew Rate (5uA/V) shows a faster recovery to the nominal output voltage after a load jump compared to the OTA with a lower Slew Rate (2uA/V). This demonstrates the importance of optimizing the Slew Rate in the design of voltage regulators, especially in applications where rapid load changes are expected.

Load jump response comparison between two OTA models with different Slew Rates

However, we must keep in mind the conditions under which eq. (1) was derived and remains valid. The equation assumes that the output current exhibits both a hard saturation on the bias current and a linear dependence on the input differential voltage. Therefore, increasing the Slew Rate requires more current from the OTA, which leads to increased power consumption. Therefore, there is a trade-off between transient response and power efficiency that must be carefully balanced in the design of the voltage regulator, for class-A type OTAs.

For class AB OTAs, which typically exhibit a nonlinear relationship between the output current and the input differential voltage, the situation differs. In these devices, the slew rate can be enhanced without significantly raising the quiescent current, improving the transient response without a substantial power penalty. dc_sweep_ota_model_linear_vs_quadratic illustrates the differing output current behaviors of a linear and a quadratic OTA model. The linear model demonstrates hard saturation at the 2 µA bias current when the output voltage drops below 1 V. Conversely, the quadratic model initially presents a lower slope, resulting in a lower transconductance for minor voltage deviations, but this slope increases rapidly. Consequently, its output current overtakes that of the linear model for voltages below 1.025 V. (It should be noted that no current saturation was incorporated into this simplified quadratic model). Therefore, boosting an OTA’s slew rate via a nonlinear stage can improve transient behavior and power efficiency, even if the unity-gain bandwidth is held constant.

DC sweep of the output current injected by the OTA block for a decrease in its negative input voltage

The resulting transient response of both ota models to a load jump is shown in loadJump_ota_model_linear_vs_quadratic. The OTA with the nonlinear output current dependence on the input differential voltage shows a smaller undershoot and a faster recovery to the nominal output voltage after a load jump compared to the OTA with a linear output current dependence, even though both OTAs have the same unity-gain bandwidth. This demonstrates that optimizing the Slew Rate through nonlinear stages can significantly enhance the transient response of voltage regulators without necessarily increasing power consumption.

Load jump response comparison between one OTA model with linear Iout dependence on input differential voltage and Slew Rate cap and another OTA model with quadratic Iout dependence on input differential voltage

## Stability Analysis of Dual Loop Systems

Analyzing the stability of systems with multiple interacting feedback loops can be challenging. Traditional methods developed for single-loop systems cannot always be directly applied and finding the right point to break all loops is not possible for all such systems. Signal Flow Graphs (SFGs) offer a powerful, visual approach to characterize linear circuit networks with loading effects at the input and output ports automatically included in the analysis [Ochoa, 1998, A systematic approach…]

An SFG visually captures the cause-and-effect relationships within a linear system and is equivalent to the Nodal Analysis Matrix Equation [Choma, Signal flow analysis…]:

I=Y⋅V

where Y is the nodal admittance matrix, V is the vector of node voltages and I is the vector of independent current sources. Mason ’s Gain formula provides a general method for finding the transfer function between two nodes once the graph has been defined. The determinant of the graph is mathematically equivalent to the determinant of the admittance matrix derived from nodal analysis (1).

Just as the admittance matrix characteristic equation d`efines the system natural frequencies, the determinant of the graph contains information about the system poles and zeroes since it defines the loop gain of the system.

The SFG together with the DPI Technique encompass an analysis method for generating the graph of the system as a direct interpretation of the circuit topology. Once the Signal Flow Graph has been found, Mason’s rule and other well-known rules for collapsing graphs can be applied to produce the desired transfer function. The technique of DPI analysis is based on the transformation of circuit ports to their Norton equivalent representation and on the sequential application of superposition [1].

These circuit analysis techniques having been used for the VREG in Fig. 1 give rise to the SFG in Fig. 2 and lead to the following loop gain equation:

LG(S)≈Gm2R21+Gm1R1f1+sC1Gm1f1+sCGSgmp1+sR2C21+sR1C11+sCLgmp−1+sCGSgmpsCGSR21+sR2C21+sCLgmp

Equation 3 indicates a three pole system with a pole associated with each of the three nodes denoted in Fig 1, the DC loop gain determined by the voltage gains of the two OTAs within the two signal paths and a zero that appears due to the feedforwarding action of the inner loop on the outer loop. Another zero appears because of the CGS of the power stage. That same parasitic capacitance, if viewed bilaterally, yields a rhp pole, as indicated by the negative term seen in (3). The following inequalities turn into design guidelines:

Firstly, looking back to why we chose this configuration in the first place, we should place a condition on the two loops bandwidths:

The DC gain we would like to be as high as possible:

To maintain stability, we need one zero in the transfer function to occur before the unity gain frequency, where the UGF has been computed as of a two-pole system with no dominant pole:

One can also obtain this inequality by following the procedure described in [3]: analyse the dual-loop system by breaking the inner loop first and then evaluating the outer loop with the inner loop replaced with its closed-loop equivalent as stated by classical feedback theory. Last but not least, to ensure a suitable phase margin, the pole associated with the output node should be pushed up in frequency at least a decade above the unity gain bandwidth of the system:

These four inequalities (4) - (7) have been put to use by designing a Dual Loop Voltage Regulator that is stable across a wide load current range: 0 - 10mA with a worst-case PM above {20}^circ as shown in Fig 3

## TODO:

- stability analysis

- evaluate different class AB OTA topologies

- Pass Device sizing